SIMULATION TASK

Question 1: LTE-Advanced Carrier Aggregation Performance

Program code:

clear; clc; close all;

%% 1. Simulation Parameters

snr_dB = 0:2:30; % SNR range in dB

snr_lin = 10.^(snr_dB / 10); % SNR in linear scale

implementation_eff = 0.6; % LTE Link Efficiency factor (~60% of Shannon capacity)

% Carrier Aggregation Configurations

% Format: [Num Carriers, Bandwidth per Carrier (MHz)]

configs = struct(...

'Single', [1, 20], ...

'Dual', [2, 20], ...

'Triple', [3, 20] ...

);

%% 2. Calculate Throughput vs SNR (Fixed Component Carrier Bandwidth = 20 MHz)

throughput_single = configs.Single(1) * (configs.Single(2) * 1e6) * implementation_eff .* log2(1 + snr_lin) / 1e6; % Mbps

throughput_dual = configs.Dual(1) * (configs.Dual(2) * 1e6) * implementation_eff .* log2(1 + snr_lin) / 1e6; % Mbps

throughput_triple = configs.Triple(1) * (configs.Triple(2) * 1e6) * implementation_eff .* log2(1 + snr_lin) / 1e6; % Mbps

%% 3. Calculate Throughput vs Varying Total Bandwidth (At Fixed SNR = 15 dB)

fixed_snr_lin = 10^(15 / 10);

total_bw_range = 10:10:100; % Total Bandwidth in MHz

throughput_vs_bw = total_bw_range * 1e6 * implementation_eff * log2(1 + fixed_snr_lin) / 1e6;

%% 4. Plotting Results

figure('Name', 'LTE-A Carrier Aggregation Performance', 'NumberTitle', 'off');

% Subplot 1: Throughput vs SNR for Single, Dual, Triple CC

subplot(2,1,1);

plot(snr_dB, throughput_single, '-o', 'LineWidth', 2, 'DisplayName', 'Single Carrier (1x20 MHz)'); hold on;

plot(snr_dB, throughput_dual, '-s', 'LineWidth', 2, 'DisplayName', 'Dual Carrier (2x20 MHz)');

plot(snr_dB, throughput_triple, '-^', 'LineWidth', 2, 'DisplayName', 'Triple Carrier (3x20 MHz)');

grid on;

title('LTE-A Throughput vs. SNR for Different CA Configurations');

xlabel('SNR (dB)');

ylabel('Throughput (Mbps)');

legend('Location', 'northwest');

% Subplot 2: Throughput vs Varying Total Aggregated Bandwidth

subplot(2,1,2);

plot(total_bw_range, throughput_vs_bw, '-d', 'LineWidth', 2, 'Color', [0.8500 0.3250 0.0980]);

grid on;

title('LTE-A Max Capacity vs. Total Aggregated Bandwidth (SNR = 15 dB)');

xlabel('Total Aggregated Bandwidth (MHz)');

ylabel('Throughput (Mbps)');

%% 5. Display Summary Metrics

disp('--- LTE-A Carrier Aggregation Analysis Summary ---');

fprintf('At 15 dB SNR:\n');

fprintf(' Single Carrier (20 MHz): %.2f Mbps\n', throughput_single(snr_dB == 15));

fprintf(' Dual Carrier (40 MHz): %.2f Mbps\n', throughput_dual(snr_dB == 15));

fprintf(' Triple Carrier (60 MHz): %.2f Mbps\n', throughput_triple(snr_dB == 15));

Output:

Question 2: NB-IoT Performance Simulation

Program code:

clear; clc; close all;

%% 1. Parameters & Coverage Classes

% Coverage Enhancement (CE) Levels: Normal, Extended, Extreme

repetitions = [1, 4, 16, 64, 128]; % Repetition count required for deep coverage

num_levels = length(repetitions);

bw_nbiot = 180e3; % 180 kHz Subcarrier Bandwidth

base_snr_dB = [10, 2, -5, -12, -18]; % Base SNR worsening with distance/penetration

payload_bits = 1000 * 8; % 1 KB payload

% Hardware Power Profiles (mW)

P_tx = 500; % Transmit Power

P_rx = 80; % Receive Power

P_idle = 2; % Idle / DRX Power

P_sleep = 0.015; % PSM (Power Saving Mode)

%% 2. Performance Calculations

% A. Throughput & Delay

base_data_rate = 20e3; % Max uplink raw rate (~20 kbps)

effective_data_rate = base_data_rate ./ repetitions; % Rate degrades with repetitions

delay_sec = (payload_bits ./ effective_data_rate) + (repetitions * 0.01); % Transmission + latency

% B. Energy & Power Consumption per Transmission Cycle

e_tx = (delay_sec .* P_tx) + (0.5 * P_rx) + (2 * P_idle); % Energy in mJ

avg_power_mW = e_tx ./ (delay_sec + 10); % Averaged over 10s reporting period

% C. Packet Loss Ratio (PLR) model based on SNR and repetitions

snr_lin = 10.^(base_snr_dB / 10);

combining_gain = 10 * log10(repetitions); % Processing gain from repetitions

effective_snr_dB = base_snr_dB + combining_gain;

% Bit Error Rate (BER) for BPSK in AWGN and estimated Packet Loss

ber = 0.5 * erfc(sqrt(10.^(effective_snr_dB/10)));

packet_loss_ratio = 1 - (1 - ber).^payload_bits;

packet_loss_ratio = min(max(packet_loss_ratio, 0.001), 0.95); % Clamped for realism

%% 3. Visualizations

figure('Name', 'NB-IoT Performance Trade-offs', 'NumberTitle', 'off');

% Subplot 1: Delay vs Repetitions (Coverage Trade-off)

subplot(2,2,1);

stem(repetitions, delay_sec, 'filled', 'LineWidth', 1.5, 'Color', 'b');

grid on; set(gca, 'XScale', 'log');

title('A. Latency/Delay vs. Repetitions');

xlabel('Repetition Count (Coverage Drive)'); ylabel('Latency (Seconds)');

% Subplot 2: Effective Throughput vs Repetitions

subplot(2,2,2);

plot(repetitions, effective_data_rate/1000, '-o', 'LineWidth', 2, 'Color', 'r');

grid on; set(gca, 'XScale', 'log');

title('B. Effective Throughput vs. Repetitions');

xlabel('Repetition Count'); ylabel('Throughput (kbps)');

% Subplot 3: Average Power Consumption

subplot(2,2,3);

bar(1:num_levels, avg_power_mW, 'FaceColor', [0.4660 0.6740 0.1880]);

grid on; set(gca, 'XTickLabel', repetitions);

title('C. Avg Power Consumption');

xlabel('Repetition Count'); ylabel('Power (mW)');

% Subplot 4: Packet Loss Ratio

subplot(2,2,4);

plot(effective_snr_dB, packet_loss_ratio * 100, '-^k', 'LineWidth', 2);

grid on;

title('D. Packet Loss vs. Effective SNR');

xlabel('Effective SNR after Combining (dB)'); ylabel('Packet Loss (%)');

%% 4. Print Analysis

disp('--- NB-IoT Coverage Level Analysis ---');

for i = 1:num_levels

fprintf('Reps: %3d | Effective SNR: %5.1f dB | Latency: %5.2f s | Energy: %6.1f mJ | Packet Loss: %4.1f%%\n', ...

repetitions(i), effective_snr_dB(i), delay_sec(i), e_tx(i), packet_loss_ratio(i)*100);

end

Output: